Digital Blossom

Smart Connected Flower Pot — Technical Specification

| Status | Active Specification |
| --- | --- |
| Author | IoT Engineering & Design Team |
| Date | May 2026 |
| Version | v1.0 |

## Table of Contents

Right-click and select 'Update Field' to populate the table of contents.

# 1. Introduction & Overview

An interactive, long-distance relationship desktop art piece and IoT device, “Digital Blossom” features a circular color IPS display mounted on a hollow metallic stem resembling a flower blossom. This is paired with a smart electronics hub, physical controls, an integrated battery charger, a secondary dashboard status display, and a two-way digital voice mailbox—all nestled inside a compact desktop planter pot.

This document details the complete firmware architecture, mechanical specifications, and assembly instructions required to build, flash, and configure the Digital Blossom system.

## 1.1 Key Features

“Touch of Love” Presence / Virtual Water Drop: A capacitive touch sensor wired to a brass leaf or pot rim detects physical interaction. Touching the leaf registers a timestamp on a shared real-time database. If your partner touches theirs within 10 seconds, both devices trigger a synchronized, vibrant “Together Now” golden blooming petal animation accompanied by a harmonic chime.

Two-Way Audio Mailbox: Record and stream 10-second digital voice notes directly between pots. Pressing and holding the rotary encoder records audio via an I2S digital microphone, packaging it as a .wav file before posting it to a Telegram channel. An incoming note flashes a classic cassette tape icon on the OLED dashboard; tapping the rotary switch plays the note through an integrated I2S Class-D amplifier and mini speaker.

Remote Photo & Art Streaming: Easily stream photos or custom illustrations to your partner’s blossom. Images uploaded to a dedicated Telegram Bot are automatically fetched, decoded in high-capacity PSRAM, and rendered directly onto the circular IPS screen.

Atmospheric & Multi-View Engine: Use the physical rotary dial to scrub through multiple system modes:

Living Blossom: Interactive real-time digital art and smooth visual animations driven by the LVGL graphics library.

Circadian Sky & Time Zone Dial: The background sky dynamically shifts (dawn, day, golden hour, starry night) based on your partner’s local time zone, calculated using a high-precision DS3231 Real-Time Clock.

Milestone Time Capsule: Scrub through custom relationship milestones (e.g., “Our First Trip — 428 Days Ago”) on the base OLED while displaying corresponding pictures on the circular LCD.

Focus Bloom: A productivity Pomodoro timer that displays a closed flower bud slowly opening petal by petal over 25 minutes.

# 2. Hardware Architecture & BOM

To successfully handle dual-buffered graphic layouts, secure webhooks, and digital audio streams, staying within the ESP32-S3 family with external PSRAM is required. Below is the complete bill of materials optimized to stay within a €30 budget.

## 2.1 Core Processor & Power

Microcontroller: ESP32-S3 DevKitC-1 N16R8 (16MB Flash / 8MB Octal PSRAM)

Battery: 3.7V XTAR 18650 Li-ion Battery (2600mAh)

Battery Holder: Single-slot 18650 plastic battery holder with pre-attached wire leads

Charger Module: TP4056 USB-C Li-ion Charging Module with Protection (must have 6 solder pads: B+, B-, OUT+, OUT-)

Power Switch: Mini SPDT slide switch (cuts battery connection during USB coding/programming)

Voltage Divider Resistors: Two 100kΩ resistors (for safe battery level monitoring via ESP32 ADC)

## 2.2 Audio & Interfaces

Visual Display (Blossom): 1.28” Round IPS LCD Display (GC9A01 driver over SPI, 240x240 resolution)

Visual Display (Dashboard): 0.91” Monochrome OLED Display (SSD1306 driver over I2C, 128x32 resolution)

Microphone (Audio In): INMP441 I2S Digital MEMS Microphone Module

Audio Amplifier (Audio Out): MAX98357A I2S Class-D 3W Amplifier Breakout

Speaker: 8 Ohm, 1W to 2W Micro Speaker (diameter ≤ 28mm to fit enclosure base)

Rotary Encoder: KY-040 Rotary Encoder Module (with integrated push-button)

Real-Time Clock: DS3231 High-Precision RTC Module

Touch Sensor: Solid brass wire, brass rod, or a decorative brass leaf

## 2.3 Structural & Prototyping

Planter Enclosure: 3D-printed, plastic, or wood desktop planter pot (with cutouts for screens, USB, and dials)

Flower Stem: Hollow metal tube (8mm to 10mm brass, copper, or aluminum pipe) to route 7 thin wires to the circular screen

Prototyping Gear: Solderless breadboard, Dupont jumper wires, 30 AWG thin solid core wire, soldering iron, and solder

# 3. ESP32-S3 System Pinout Mapping

The abundant GPIO capacity of the ESP32-S3 DevKitC-1 allows all components to connect simultaneously. Refer to the hardware mapping table below for wiring configurations:

| Peripheral / Component | Pin Name | GPIO | Bus / Function |
| --- | --- | --- | --- |
| GC9A01 SCL (SCK) | D8 | GPIO 9 | Hardware FSPI SCK |
| GC9A01 SDA (MOSI) | D10 | GPIO 10 | Hardware FSPI MOSI |
| GC9A01 CS | D1 | GPIO 2 | SPI Chip Select |
| GC9A01 DC | D2 | GPIO 3 | Data / Command |
| GC9A01 RES | D0 | GPIO 1 | Display Hardware Reset |
| Shared I2C SDA | D4 | GPIO 5 | SSD1306 (0x3C) & DS3231 (0x68) SDA |
| Shared I2C SCL | D5 | GPIO 6 | SSD1306 (0x3C) & DS3231 (0x68) SCL |
| Rotary CLK | D6 | GPIO 7 | Encoder Phase A |
| Rotary DT | D7 | GPIO 8 | Encoder Phase B |
| Rotary SW | D9 | GPIO 44 | Push-button (Internal Pullup, Active LOW) |
| Capacitive Touch | D3 | GPIO 4 | Brass Leaf Capacitive Touch Interrupt |
| MAX98357A DIN | D13 | GPIO 13 | I2S Channel 0: Serial Data Out |
| MAX98357A BCLK | A0 | GPIO 15 | I2S Channel 0: Bit Clock |
| MAX98357A LRCLK | A1 | GPIO 16 | I2S Channel 0: Word Select (Left/Right Clock) |
| INMP441 SD | D11 | GPIO 11 | I2S Channel 1: Serial Data In |
| INMP441 SCK | D12 | GPIO 12 | I2S Channel 1: Bit Clock |
| INMP441 WS | D14 | GPIO 14 | I2S Channel 1: Word Select |
| Battery Gauge ADC | A4 | GPIO 17 | Analog voltage read via 100kΩ/100kΩ divider |

# 4. Power Distribution & Charging

Ensure your XTAR 18650 cell is routed through the TP4056 protection board. Use a slide switch to protect your computer’s USB port from power feedback during programming.

+------------------------------------+ ¦ TP4056 Charger with Protection ¦ ¦ ¦ ¦ [USB-C Input] B+ B- OUT+ OUT-¦ +-----------------¦-----¦----+----+--+ ¦ ¦ ¦ ¦ +----------------------+ ¦ ¦ ¦ ¦ ¦ ¦ ¦ (Red Wire +) ¦ ¦ ¦ +-------------+ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ 18650 Cell ¦ ¦ ¦ ¦ ¦ (Battery) ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ +-------------+ ¦ ¦ ¦ (Black Wire -) ¦ ¦ ¦ +----------------------------+ ¦ ¦ ¦ ¦ [Slide Switch] ¦ ¦ v v (5V) (GND) ¦ ¦ v v ESP32-S3 Board

Figure 1: Power Distribution and Charging Schematic

# 5. Firmware Setup & Development

The software architecture is split across both CPU cores on the ESP32-S3 to prevent networking routines from blocking user interface animations.

## 5.1 Core Allocation

Core 0 (Network & Background Daemons): Handles WiFiManager setups, secure Telegram Bot long-polling/Firebase endpoints, file synchronization in LittleFS, and RTC time updates.

Core 1 (UI Engine & Interaction): Drives the round GC9A01 display using LVGL (v8.3+) with hardware DMA acceleration, renders the OLED dashboard using the U8g2/Adafruit libraries, handles rotary interrupts, and decodes incoming I2S audio streams.

## 5.2 Required Software Libraries

Install the following libraries using the Arduino IDE Library Manager (Ctrl+Shift+I):

TFT_eSPI: Optimized SPI library for the GC9A01 LCD.

Adafruit_SSD1306 & Adafruit_GFX: For driving the dashboard OLED.

UniversalTelegramBot & ArduinoJson: For parsing Telegram messages, photos, and voice notes.

RTClib: For synchronization and reading from the DS3231 RTC.

lvgl (v8.3.x): Embedded Light and Versatile Graphics Library for the blossom interface.

## 5.3 Compiling and Verifying

To check code syntax before your components arrive, follow these steps:

Open your Arduino IDE.

Go to File > Preferences and add the following URL to the Additional Boards Manager URLs field:
https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

Go to Tools > Board > Boards Manager, search for esp32, and install the library package.

Set your board target to ESP32-S3 DevKitC-1 under Tools > Board.

In the settings, ensure PSRAM is configured as OPI PSRAM to match the WROOM-1 N16R8 module specifications.

Verify your code by clicking the Verify (Checkmark) icon on the top-left of the IDE interface.

# 6. Getting Started: Tonight's Action Plan

Do not wait for your physical components to arrive to get started on the Digital Blossom! Follow these immediate steps to lay the groundwork:

Secure Your Cloud Space: Contact the @BotFather on Telegram, create your custom bot, and save your secure API key. Use the @IDBot to retrieve your Chat ID.

Setup Your Environment: Install the Arduino IDE, download the Espressif ESP32 package, and pre-install the libraries listed in Section 5.2. Write the core connection and JSON skeleton files.

Layout Enclosure Dimensions: Draft your planter pot design on paper or in 3D CAD software (such as Tinkercad or Fusion360) ensuring the base compartment is at least 85mm wide to accommodate the 18650 holder and DevKitC-1.

Draft Your Assets: Write your 365 daily quotes, crop relationship pictures into 240x240 circular crops, and test layout grids.